Instruction manual for

RPI Tank Controller

Contents

Wiring diagram	2

Prerequisite	2

Install wifi library package	3

Enable SPI interface on RPI	4

Install Tank controller program	7

Install python Flask and configure auto start	7

Configure auto start of python source	8

Install Adafruit GPIO library	9

Install Adafruit MAX31885 library	9

Configure as Kiosk mode	9

# Wiring diagram

# Prerequisite

First, we have to install Raspbian Jessie on RPI 3B.

If you don’t have Raspbian Jessie image not Raspbian Stretch, I can send you.

And then we have to enable SSH on RPI3B.

There are several ways to enable ssh on rapbian.

Because, we are using 7” TFT touch LCD, you can configure on Touch LCD.

And then, you have to get IP address of RPI. In my testing, ip address is 192.168.1.140.

Reboot RPI.

# Install wifi library package

Connect to RPI using putty

I am using putty on windows.

In terminal(putty), we have to enter “sudo pip install wifi” command.

# Enable SPI interface on RPI

sudo raspi-config

Reboot again.

# Install Tank controller program

git clone https://github.com/arianlegend/tankcontroller.git

# Install python Flask and configure auto start

Connect to RPI using putty again.

And then, enter “sudo pip install python Flask”.

## Configure auto start of python source

In putty terminal, enter sudo nano /etc/rc.local

Before “exit 0”, add following commands.

sudo python /home/pi/tankController/tankController.py

Save it to press Ctrl + O.

Exit form that to press Ctrl + X.

# Install Adafruit GPIO library

In terminal, we have to enter following commands

git clone https://github.com/adafruit/Adafruit_Python_GPIO.git

cd Adafruit_Python_GPIO

sudo python setup.py install

# Install Adafruit MAX31885 library

git clone https://github.com/adafruit/Adafruit_Python_MAX31855.git

cd Adafruit_Python_MAX31855/

sudo python setup.py install

# Configure as Kiosk mode

So that program will auto run with full screen, when rpi boots.

sudo nano /home/pi/.config/lxsession/LXDE-pi/autostart

Edit as follows and save it.

Exit and reboot.

So, program will start automatically when RPI boots.

Gui will be displayed on your RPI, backend for control logic will be performed.